At the beginning of the process, we have two different sources of inputs:

- Inputs coming from SCADA system giving N situation of the network (compatible with Dynawo format).

- Inputs giving the list of events (faults in our case) to simulate N-1 situations of the network.

Base inputs example

IEEE14/
├── IEEE14_CRV.xml		→ Variables to plot
├── IEEE14_DYD.xml		→ Dynamic behavior

├── IEEE14_FSV.xml		→ Final state values
├── IEEE14_IIDM.xml		→ Grid description
├── IEEE14_JOBS.xml		→ General settings
├── IEEE14_PAR.xml		→ Simulation parameters
├── IEEE14_SOLVERS.xml	→ Solver
└── events/				→ Events description
       ├── Line_1-2.xml
       └── Line_1-5.xml

For each event, to launch Dynawo simulation, we need to add specific DYD and PAR files to set the characteristics of the event, plus a MULTIPLE file (one file for all events) to set the links between files. Other files don’t have to be modified. As a result, before launching EEAC, we need to create temporary files according to the events to simulate.

Final inputs example when all temporary files have been created

IEEE14/
├── IEEE14_CRV.xml
├── IEEE14_DYD.xml

├── IEEE14_DYD_Line_1-2.xml	→ DYD file for Line_1-2 event

├── IEEE14_DYD_Line_1-5.xml	→ DYD file for Line_1-5 event

├── IEEE14_FSV.xml
├── IEEE14_IIDM.xml
├── IEEE14_JOBS.xml

├── IEEE14_MULTIPLE.xml	→ MULTIPLE file
├── IEEE14_PAR.xml

├── IEEE14_PAR_Line_1-2.xml	→ PAR file for Line_1-2 event

├── IEEE14_PAR_Line_1-5.xml	→ PAR file for Line_1-5 event
├── IEEE14_SOLVERS.xml
└── events/
       ├── Line_1-2.xml
       └── Line_1-5.xml

NB:

- Complete process using EEAC: Export N-situation + list of 200 faults → use EEAC to filter non risky faults → launch Dynawo on remaining faults → display alerts if risky faults still remain. Thats is why we need to build complete Dynawo structures for each fault.

- Further developments will be needed to delete temporary files depending on process and user needs.